ChE 344

Chemical Reaction Engineering

Winter 1999

# Exam I

Part 1 (80%)

Closed Book, Notes, Disk, and Web

Name_______________________________

Honor Code________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

1)	____/10 pts

2)	____/30 pts

3)	____/20 pts

4)	____/10 pts

5)	____/10 pts

6)	____/__ pts

Total	____/80 pts

(10 pts)	1)	The trimerization

3A(g)  A3(g,l)

is carried out isothermally and without pressure drop in a PFR at 298 K and 2 atm. As the concentration of A3 increases down the reactor and A3 begins to condense. The vapor pressure of A3 at 298 K is 0.5 atm. If an equal molar mixture of A and inert, I, is fed to the reactor at what conversion of A will A3 begin to condense?

(30 pts)	2)	In order to study the photochemical decay of aqueous bromine in bright sunlight, a small quantity of liquid bromine was dissolved in water contained in a glass battery jar and placed in direct sunlight. The following data were obtained:

| Time (min) | 10 | 20 | 30 | 40 | 50 | 60 |
| --- | --- | --- | --- | --- | --- | --- |
| ppm Br | 2.45 | 1.74 | 1.23 | 0.88 | 0.62 | 0.44 |

- Determine whether the reaction rate is zero-, first-, or second-order in bromine, and calculate the reaction rate constant in units of your choice.

b)	Assuming identical exposure conditions, calculate the required hourly rate of injection of bromine (in pounds) into a sunlit body of water 25,000 gal in volume in order to maintain a sterilizing level of bromine of 1.0 ppm.

(Note: ppm parts of bromine per million parts brominated water by weight. In dilute aqueous solutions, 1 ppm=1 milligram per liter, molecular weight of Br = 80 Daltons.)

1 gal  3.785 liters

1 lb  454 gms

(20pts)	3)	The irreversible elementary gas phase reaction

2AB+C

is currently carried out in a packed bed reactor containing 100 kg of catalyst. The entering pressure is 20 atm and the exit pressure is 4 atm. Currently 50% conversion is achieved. It is proposed to add a CSTR with 200 Kg of catalyst downstream of the PBR. There is no pressure drop in the CSTR. The flow rate and temperature remain unchanged.

- What would be the overall conversion in such an arrangement?

b)	Is there a better way to carry out the reaction, and if so what is it?

## Current

### Proposed

(10 pts)	4)	The following reactions were found to occur while trying to make a desired product B

(1)AB−rA1=k1ACA2AX−rA2=k2ACAA+XY−rA3=k3ACACX

Species X and Y are both foul pollutants

- What is the instantaneous selectivity of B with respect to the foul pollutants X and Y?

b)	How would you carry out this reaction to maximize the formation of B?

Additional Information

k1A = .5 e–10,000/T min–1, T in degrees Kelvin

k2A = 50 e–20,000/T min–1, T in degrees Kelvin

k3A = 100 e–5,000/T min–1, T in degrees Kelvin

(10 pts)	5)	The catalytic reaction

AB

to be carried out in a flow reaction system has the following rate law,

−rA=kCA1+KACA2

where	k = 1 min–1

KA = 1 dm3/mol

The entering concentration of A is 2 mol/dm3. What type of reactor or combination of reactors would have the smallest volume to

- achieve 50% conversion?

- achieve 80% conversion?