ChE 344

Chemical Reaction Engineering

Winter 1999

# Mid Term Exam

Open Book, Notes, Disk, and Web

Name_______________________________

Honor Code________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

___________________________________________________________________________

1)	____/	5 pts

2)	____/	6 pts

3)	____/	6 pts

4)	____/	4 pts

5)	____/	5 pts

6)	____/	5 pts

7) 	____/	15 pts

8)	____/	10 pts

9)	____/	9 pts

10)	____/	10 pts

11)	____/	25pts

Total	____/	100pts

At the End of the Exam Turn Off Your Computer

(5 pts)	1)	The isomerization of A to B was carried out in a packed bed reactor. The catalyst does not decay. The following profiles were obtained

Circle the correct true (T) or False (F) answer for this system

T	F	a)	The above profiles could represent an adiabatic system where the addition of inerts to the feed stream will increase the conversion.

T	F	b)	A small decrease in the flow rate will produce a small increase in the conversion.

T	F	c)	An increase in the feed temperature will always increase the conversion.

T	F	d)	A decrease in feed temperature will always increase the conversion.

T	F	e)	There could be a heat exchanger attached to the reactor with the heat flow given by

dQdW=400kJkg min KT−400

(6 pts)	2)	The isomerization of A to B was carried out adiabatically in a packed bed reactor. The catalyst does not decay.

Sketch the corresponding temperature conversion profiles for A, B, and C.

(6 pts)	3)	The series reaction

ABC

is carried out in a packed bed reactor. The following profiles were obtained.

Circle the correct true (T) or False (F) answer for this system

T	F	a)	The above profiles could represent a system where the reactions are carried out adiabatically.

T	F	b)	The above profiles could represent a system where there is a heat exchanger attached to the system.

T	F	c)	The above profiles could represent an adiabatic system where both of the reactions could be endothermic.

T	F	d)	The above profiles could represent an adiabatic system where only one of the reactions is exothermic

T	F	e)	The above profiles could represent a system where the addition of inerts could decrease the exit molar flow rate of the desired product, B.

T	F	f)	The above profiles could represent an adiabatic system where increasing the feed temperature will increase the concentration of B in the exit stream.

(4 pts)	4)	The irreversible gas phase reaction of A and B to form C and D was carried out in a packed bed reactor in which there is no catalyst decay.

The following figure shows the rate of reaction at the reactor entrance as a function the partial pressure of C for various entering temperatures, To

Circle the correct answer True (T), False (F), or Cannot Tell (CT) from the information given for the above system

T	F	CT	a)	The reaction is exothermic.

T	F	CT	b)	The reaction is endothermic.

T	F	CT	c)	Species C is adsorbed on the catalyst surface at 400°K.

T	F	CT	d)	Species C is adsorbed on the catalyst at 700°K.

(5 pts)	5)	Circle the correct true (T) or False (F) answer

T	F	a)	Multiple steady states can exist for an irreversible endothermic first order reactions.

T	F	b)	Multiple steady states can only exist for reversible reactions.

T	F	c)	Multiple steady states can only exist for adiabatic reactions.

T	F	d)	Reactor staging is used for irreversible reactions.

T	F	e)	The effects of pressure drop are more pronounced for adiabatic- exothermic reactions than for adiabatic endothermic reactions.

(5 pts)	6)	The reactions

1AB+C2AD+E3A+CF+G

are carried out in a packed bed reactor where B is the desired product. The flowrate of species B exiting the reaction is shown below as a function of the entering temperature, To

Circle the correct true (T) or False (F) answer

T	F	a)	The above figure could represent an adiabatic system where the reaction 1 is adiabatic exothermic and reversible.

T	F	b)	The above figure could represent an adiabatic system where the reaction 1 is adiabatic, endothermic and reversible.

T	F	c)	The above figure could represent an adiabatic system where all reactions are endothermic.

T	F	d)	The above figure could represent a system where the reactions 1 and 3 are endothermic and reaction 2 is exothermic.

T	F	e)	The above figure could represent a system where the reactions 1 and 2 are endothermic and reaction 3 is exothermic.

(15 pts)	7)	The catalytic reaction

A+BC+D

was carried out in a differential reactor with the following results

- What is the rate law consistent with the experimental data?

- What is the mechanism and rate limiting step consistent with the rate law? (Can be done by inspection).

(10 pts)	8)	The irreversible reaction

A+BC+D

is carried out in a CSTR. The “heat generated” [G(T)] and the “heat removed” [R(T)] curves are shown below

- What is the HRx of the reaction?

HRx = 	cal/mol

- What are the ignition and extinction temperatures?

Ignition = 	°C

Extinction = 	°C

- What are the conversions just before the ignition and extinction temperatures?

X (Ignition) =

X (Extinction) =

(9 pts)	9)	Reconsider the example 9-2

- Which of the following curves best describes the correlation between the shortest length of time the heat exchange stopped functioning (i.e. Q = 0) (down time) and the time in the evening at which the malfunction occurred that will cause an explosion.

Ans

- If 10% more water had been added to the mixture in the reactor the explosion would not have occurred that evening.

True	False

(10 pts)	10)	The vapor-phase cracking of gas-oil in Example 10-7 is carried out isothermally over a different catalyst, for which the rate law is

−rA'=atk'PA2withk'=1.0´10±5kmolkgcat.⋅s⋅atm2at400°CandwithEA=5000calmol

The decay law is

−dadt=kda2PB

with kd=0.002s−1atm−1at400°CandwithED=35000cal/mol

Assuming that you can vary the entering temperature between 200°C and 700°C, what entering temperature would you recommend? (25°C)

To = 	°C

You can make any assumptions that were made in the example problem.

(25 pts)	11)	The elementary liquid phase reactions

1A+2B2C2A+C2D

are carried out adiabatically in a 10 dm3 PFR. After streams A and B mix, species A enters the reactor at a concentration of CA0 = 2 mol/dm3 and species B at a concentration of 4 mol/dm3. The entering volumetric flow rate is 10 dm3/s.

Assuming you could vary the entering temperature between 300K and 600 K, what entering temperture would you recommend to maximize the concentration of species C exiting the reactor? (25°K).

To = 	°C

Assume all species have the same density.

Additional Information

CPA=CPB=20cal/mol/KCPC=60cal/mol/KCPD=80cal/mol/KDHRx1A=20,000cal/molADHRx2A=-10,000cal/molAk1A=0.001dm6mol2⋅sat300KwithE=5000cal/molk2A=0.001dm3mol⋅sat300KwithE=7500cal/mol

Note:

Before beginning write your POLYMATH program below in POLYMATH notation

1.

2.

Continue